Division of Public Health

Lead and Asbestos Section

Work Specification Language for Lead Hazard Reduction

This list of work specifications may be used in risk assessment reports and in orders to provide options for eliminating identified lead hazards. Select the appropriate specifications then copy and paste into the risk assessment report or into work orders.

To add a work spec box in the risk assessment report:

Select Insert from the main toolbar, click on ‘Table’ then ‘Insert Table’ then set the number of columns and rows to 1 and click ‘Autofit’ to contents.

To delete box from risk assessment report:

Highlight text and click ‘Delete,’ move cursor to box outline until solid arrow appears, right click mouse and select ‘Delete Table.’

CLEANING

CLEANING HARD SURFACE (Interim Control)

Using [select options for cleaning floors or windows: a hard surface attachment for floors; an upholstery attachment for window sills and troughs; a crevice attachment for seams, joints and right angle corners], slowly HEPA vacuum areas. Thoroughly wash all hard surfaces using disposable wet wipe cloths. Wash no more than 40 square feet of floor per wet wipe and not more than one sill or trough per wet wipe. Repeat HEPA vacuuming again after surfaces have dried. OPTION: Use a two bucket wet washing method in place of disposable wet wipes.

CLEANING CARPET (Interim Control)

Using a beater bar attachment, slowly HEPA vacuum the carpeting in two different directions. Steam clean carpeting and HEPA vacuum a second time after carpet has dried.

DOORS

DOOR ADJUST (Interim Control)

Using lead-safe work practices, plane door edges and adjust the strike plate if needed to minimize door/jamb friction and contact. Door is to operate smoothly.

STABILIZE AND REPAINT INTERIOR/EXTERIOR DOOR (Interim Controls)

Using lead-safe work practices, wet scrape all loose, peeling, cracked, or blistered paint from door and door components including casing, jamb, and stop. Feather edges with a wet sponge sanding block. HEPA vacuum and wash with a de-glossing solution all surfaces to be repainted to ensure a good bond with the new paint. Apply high-quality bonding primer. Apply high-quality interior latex paint or exterior latex or acrylic paint on exterior surfaces and components that has a first coat wet film of not less than 6 mils.

STORM/SCREEN DOOR REPLACE (Abatement)

Using lead-safe abatement practices, remove and properly dispose of the existing storm/screen door. Install a new storm/screen door sized to fit the door opening.

REPLACE ENTRANCE DOOR—PREHUNG (Abatement)

Using lead-safe abatement practices, remove and properly dispose of the existing door, jamb, and threshold. Supply and install a new pre-hung exterior entrance door measured to fit existing opening, primed and finish-coat ready. Inside door casing is to be [select based on condition of existing casing: reused, replaced]. Apply high-quality exterior latex or acrylic top coat paint to jamb and brick mold trim or wrap trim with aluminum cladding. Finish door face as necessary per manufacturer instructions.

CONTINUED

REPLACE PRIVACY/CLOSET DOOR—PRE-HUNG (Abatement)

Using lead-safe abatement practices, remove and properly dispose of the existing door and jamb. Supply and install a new pre-hung interior door measured to fit existing opening. Door casing is to be [select based on condition of existing casing: reused, replaced]. Apply high-quality bonding primer. Apply high-quality interior latex top coat paint.

ENCLOSE

ENCLOSE/WRAP EXTERIOR TRIM WITH ALUMINUM (Abatement)

Using lead-safe abatement practices, replace any missing or rotten trim with dimensional pine or fir stock as needed. Wrap all exposed exterior window and door trim, sills, beams, and headers with aluminum coil stock custom bent to cover the component. Secure with nails and seal all edges and seams with an exterior silicon caulk.

ENCLOSE/WRAP FASCIA WITH ALUMINUM (Abatement)

Using lead-safe abatement practices, replace any missing or rotten trim with dimensional pine or fir stock as needed. Wrap all fascia with aluminum coil stock custom bent to cover fascia and fascia trim molding. Secure with nails and seal all edges and seams with an exterior silicon caulk.

ENCLOSE SOFFIT WITH ALUMINUM (Abatement)

Using lead-safe abatement practices, replace any missing or rotten trim with dimensional wood. Install solid aluminum soffit panels. If soffit is vented, install vented aluminum soffit panels in locations of existing vents.

ENCLOSE INTERIOR WALL/CEILING (Abatement)

[Edit for addressing walls or ceiling] Using lead-safe abatement practices, install wood paneling, wainscot, or not less than 3/8-inch drywall over the existing surface material. Apply a continuous bead of caulk or construction adhesive around all four edges of each panel or sheet. Secure in place with nails or drywall screws—8 inches on center. Seal all edges with a high-quality, paintable silicone caulk. If the baseboard has a cap molding, use lead-safe work practices to remove and properly dispose of the top cap molding. Install new quarter round or other type of molding as a cap after drywall is paint-ready.

ENCLOSE EXTERIOR CEILING (Abatement)

Using lead-safe abatement practices, install solid aluminum soffit panels. Apply high-quality paintable silicone caulk to seal all edges.

FLOORS (See also cleaning carpets)

SEAL CONCRETE FLOOR (Interim Control)

HEPA vacuum slowly in two directions. Prepare concrete per manufacturer’s instructions. Apply concrete sealer per manufacturer’s instructions to seal concrete pores.

SEAL HARDWOOD FLOOR (Interim Control)

Prepare floor surface per manufacturer instructions. Using the crevice attachment, slowly HEPA vacuum each seam between individual boards. Using the hard floor attachment, slowly HEPA vacuum the entire floor surface in two directions. Apply high-quality polyurethane or acrylic hardwood floor finish sealer per manufacturer instructions.

REFINISH HARDWOOD FLOOR (Abatement)

Using lead-safe abatement practices, including HEPA filtered exhaust dust collection, sand the flooring down to bare wood. Using the crevice attachment, slowly HEPA vacuum each seam between individual boards. Using the hard floor attachment, HEPA vacuum the entire floor surface slowly in two directions. Apply high-quality polyurethane or acrylic hardwood floor sealer per manufacturer’s instructions. OPTION: apply a lead-free floor stain or finish prior to the floor finish sealer.

CONTINUED

STABILIZE AND PAINT PLANK FLOOR (Interim Controls)

Using lead-safe work practices, repair all loose or damaged [select option to match site conditions: flooring, stair treads or stair risers, or, wet-scrape flooring, treads and risers] to remove all loose, peeling, blistered, or flaking paint. Feather all edges with a wet sponge sanding block. HEPA vacuum all components. Apply high-quality bonding primer if required. Apply high-quality latex or acrylic interior floor paint per manufacturer’s instructions. The paint used must be specifically for interior floors or designed to withstand foot traffic.

FLOOR RUNNER/STAIR PROTECTOR (Interim Control)

Using a hard surface attachment for floors and crevice attachment for seams, joints, and right angle corners, slowly HEPA vacuum surfaces. Thoroughly wash all surfaces using disposable wet wipe cloths. Wash no more than 40 square feet per wet wipe. Repeat HEPA vacuuming again after surfaces have dried. [select appropriate option: Securely install a vinyl or rubber floor runner in high-traffic areas; or, secure tread protectors on all stair treads.]

PLANK FLOOR (Abatement)

Using lead-safe abatement practices, remove and properly dispose of baseboard or bottom shoe molding. Slowly HEPA vacuum all floors. Using a crevice attachment, slowly HEPA vacuum each seam and gap between individual floor boards. Install not less than ¼-inch underlayment by applying a continuous bead of caulk or construction adhesive around all four edges of each sheet to form a dust-tight seal. Following manufacturer recommendations, nail, staple, or screw secure sheets to existing flooring. Install new baseboard or base molding. (Note: Lead-safe work practices for baseboards are only required if coated with lead-based paint or unknown and assumed to contain lead-based paint.)

GUTTERS

REPLACE OR INSTALL GUTTERS/DOWNSPOUTS (Abatement)

Using lead-safe abatement practices, remove and properly dispose of the existing gutters and downspouts. Install new aluminum gutters with all joints sealed water tight. Install new aluminum downspouts secured to the house with not less than 4-foot flip-ups to direct water away from the foundation.

PAINT/VARNISH COMPONENTS (See also replace and paint component, siding and windows)

PREP AND PAINT EXTERIOR TRIM (Interim Control)

Using lead-safe work practices, repair or replace all areas of damaged or rotten wood. Wet scrape all loose, peeling, cracked, or blistered paint from trim. Feather edges with a wet sponge sanding block. HEPA vacuum and wash with a de-glossing solution all surfaces to be repainted to ensure a good bond with the new paint. Apply high-quality bonding primer. Apply high-quality exterior latex or acrylic paint that has a first coat wet film of not less than 6 mils.

PREP AND PAINT FASCIA (Interim Control)

Using lead-safe work practices, repair or replace all areas of damaged or rotten wood. Wet scrape all loose, peeling, cracked, or blistered paint from fascia and fascia trim. Feather edges with a wet sponge sanding block. HEPA vacuum and wash with a de-glossing solution all surfaces to be repainted to ensure a good bond with the new paint. Apply high-quality bonding primer. Apply high-quality exterior latex or acrylic paint that has a first coat wet film of not less than 6 mils.

PREP AND PAINT SOFFIT (Interim Control)

Using lead-safe work practices, repair or replace missing or rotten wood. Wet scrape all loose, peeling, cracked, or blistered paint from soffit. Feather edges with a wet sponge sanding block. HEPA vacuum and wash with a de-glossing solution all surfaces to be repainted to ensure a good bond with the new paint. Apply high-quality bonding primer. Apply high-quality exterior latex or acrylic paint that has a first coat wet film of not less than 6 mils.

CONTINUED

PREP AND PAINT INTERIOR TRIM (Interim Control)

Using lead-safe work practices, repair or replace all areas of damaged or rotten wood. Wet scrape all loose, peeling, cracked, or blistered paint from trim. Feather edges with a wet sponge sanding block. HEPA vacuum and wash with a de-glossing solution all surfaces to be repainted to ensure a good bond with the new paint. Apply high-quality bonding primer. Apply high-quality interior latex paint that has a first coat wet film of not less than 6 mils.

PREP AND PAINT EXTERIOR CONCRETE (Interim Control)

Using lead-safe work practices, wet-scrape all loose, peeling, cracked, or blistered paint from surface. HEPA vacuum the surface to be repainted. Apply high-quality exterior concrete or masonry paint in accordance with the manufacturer’s instructions.

PREP AND PAINT INTERIOR CONCRETE (Interim Control)

Using lead-safe work practices, wet scrape all loose, peeling, cracked, or blistered paint from surface. HEPA vacuum the surface to be repainted. Apply high-quality concrete or masonry paint in accordance with the manufacturer’s instructions.

PREP AND VARNISH INTERIOR TRIM (Interim Control)

Using lead-safe work practices, repair or replace all areas of damaged or rotten wood. Wet scrape all loose, peeling, cracked, or blistered varnish from trim. Lightly wet sand varnish surface to ensure a good bond with new varnish. HEPA vacuum and then wipe with a tack cloth all surfaces to be varnished. Apply two coats of high-quality, low-odor, fast-drying polyurethane varnish following manufacturer instructions for surface preparation between coats.

PREP AND PAINT INTERIOR WALL/CEILING (Interim Control)

Using lead-safe work practices, wet scrape all loose, cracked, peeling, or blistered surfaces. Repair all cracks, holes, or deteriorated areas with joint compound or plaster patching compound. HEPA vacuum and wash with a de-glossing solution all surfaces to be painted. Apply high-quality bonding primer. Apply high-quality interior latex paint.

PREP AND PAINT EXTERIOR CEILING (Interim Control)

Using lead-safe work practices, wet scrape all loose, cracked, peeling, or blistered surfaces. Repair all cracks, holes, or rotted areas. HEPA vacuum and wash with a de-glossing solution all surfaces to be painted. Apply high-quality bonding primer. Apply high-quality exterior latex or acrylic paint that has a first coat wet film of not less than 6 mils.

PORCH/DECK COMPONENTS

STABILIZE AND PAINT DECK/PORCH (Interim Control)

Using lead-safe work practices, repair all loose or damaged [select to match findings: decking, safety railing, stair treads, stair risers.] Wet scrape the [deck, steps and safety railing] to remove all loose, peeling, blistered, or flaking paint. Feather all edges with a wet sponge sanding block. HEPA vacuum all components and apply high-quality bonding primer. Apply high-quality exterior latex or acrylic deck paint per manufacturer’s instructions. The paint used must be specifically for exterior decks or designed to withstand foot traffic outside.

REPLACE COMPONENTS ON EXTERIOR DECK/PORCH (Abatement)

Using lead-safe abatement practices, remove and properly dispose of [select components to match findings: decking, stair treads, risers, stringers]. Install new dimensional construction grade lumber or apply a chemical paint stripper that does not contain methylene chloride to remove all layers of paint to bare wood. Apply high-quality bonding primer and a high-quality exterior latex or acrylic deck paint. The paint used must be specifically for exterior decks or designed to withstand foot traffic outside. Follow manufacturer instructions if using green pressure-treated lumber that will be painted or stained.

CONTINUED

REPAINT COLUMN/POST (Interim Control)

Using lead-safe work practices, repair or replace all areas of damaged or rotten wood. Wet scrape all loose, peeling, cracked, or blistered paint from columns. Feather edges with a wet sponge sanding block. HEPA vacuum and wash with a de-glossing solution all surfaces to be repainted to ensure a good bond with the new paint. Apply high-quality bonding primer. Apply high-quality exterior latex or acrylic paint that has a first coat wet film of not less than 6 mils.

REPAINT COLUMN/POST (Abatement)

Using lead-safe abatement practices, repair or replace all areas of damaged, missing, or rotten wood. Apply a chemical paint stripper that does not contain methylene chloride to remove all layers of paint to bare wood. Apply high-quality bonding primer and a high-quality exterior latex or acrylic paint that has a first coat wet film of not less than 6 mils.

REPLACE COLUMN/POST (Abatement)

Using lead-safe abatement practices, remove and properly dispose of existing columns. Install new columns using construction grade dimensional lumber. Apply high-quality bonding primer. Apply high-quality exterior latex or acrylic paint that has a first coat wet film of not less than 6 mils. OPTION: Use dimensional green pressure-treated lumber and follow manufacturer’s instructions for applying a finish.

REPLACE AND PAINT COMPONENT

REPLACE AND PAINT EXTERIOR TRIM (Abatement)

Using lead-safe abatement practices, remove and properly dispose of existing trim. Install new trim using construction grade dimensional lumber or manufactured mill stock. Apply high-quality bonding primer. Apply high-quality exterior latex or acrylic paint that has a first coat wet film of not less than 6 mils.

REPLACE AND PAINT INTERIOR TRIM (Abatement)

Using lead- safe abatement practices, remove and properly dispose of existing trim. Install new trim using construction grade dimensional lumber or manufactured mill stock. Apply high-quality bonding primer. Apply high-quality interior latex paint that has a first coat wet film of not less than 6 mils.

REPLACE AND VARNISH INTERIOR TRIM (Abatement)

Using lead-safe abatement practices, remove and properly dispose of existing trim. Install new trim using construction grade dimensional lumber or mill stock. Apply a lead-free interior stain. Apply high-quality, low-odor, fast-drying polyurethane varnish.

ROOFING

ROOF (If roof work is needed, select the appropriate option below)

If there is evidence of water damage on the ceilings or walls in the home, and the damage was not caused by a plumbing leak inside the house, repair or replacement of the roof is required. Any current or potential source of water damage must be corrected to prevent deterioration of the substrate.

REPLACE MISSING OR DAMAGED ROOF SHINGLES

Remove damaged shingles and install new shingles, ridge, or hip capping in all areas where shingles or capping is missing or was removed.

REPLACE ROOF FLASHING

Remove and properly dispose of leaking [edit to match findings: chimney, valley, step flashing, vent boots]. Install new [chimney, valley, step flashing, vent boot] and seal to provide a leak-tight fit.

CONTINUED

REPLACE ROOF

Remove and properly dispose of existing roofing. Certification and lead-safe work practices are required if gutters and edging are coated with lead-based paint. Replace any rotten or damaged decking or sheathing. Replace all flashings, and vent boots if deteriorated or damaged in any way. Install 15 lb. felt and self-sealing 230 lb. shingles with not less than a 20-year warranty. Reinstall gutters or install new gutters.

SIDING

PAINT SIDING (Interim Control)

Using lead-safe work practices, repair or replace all areas of damaged, missing, or rotten siding. Wet scrape all loose, peeling, cracked, or blistered paint from siding. Feather edges with a wet sponge sanding block. HEPA vacuum and wash with a de-glossing solution all surfaces to be repainted to ensure a good bond with the new paint. Apply high-quality bonding primer. Apply high-quality exterior latex or acrylic paint that has a first coat wet film of not less than 6 mils.

REPLACE SIDING (Abatement)

Using lead-safe abatement practices, remove and properly dispose of the existing siding. Install house wrap over sheathing following manufacturer’s instructions. Install insulation board if necessary and install new vinyl or metal siding. OPTION TO ENCLOSE SIDING: Bend a U channel to match the thickness of insulation board. Outside lip of channel should not be less than ½-inch high. Install channel along the bottom edge of existing siding and seal to existing siding with duct tape or another brand of construction grade adhesive tape. Install house wrap over existing siding extending into the U channel. Seal all seams and edges with tape designed for use with the house wrap. Install insulation board seated inside the U channel. Seal outside lip of channel to insulation board using duct tape or another brand of construction grade adhesive tape. Install new vinyl or metal siding.

SOIL

SOIL (Interim Control)

HEPA vacuum all paint chips on the ground. Install a barrier of not less than 3 inches of mulch or install new topsoil that has a lead concentration of less than 400 parts per million (ppm). Thoroughly tamp new soil to prevent settling. Soil cultivation or rototill mixing is not allowed to lower concentration.

SOIL (Abatement) (If soil tested greater than 5,000 ppm or is located in a play area it should be abated. Interim controls are not recommended.)

Remove and properly dispose of the top 6 inches of existing soil. Soil must be analyzed at a laboratory capable of doing a Toxicity Characteristic Leaching Procedure (TCLP). If necessary, treat soil with a reagent product to permanently stabilize the lead resulting in a TCLP less than 5 parts per million (ppm). Install new topsoil thoroughly tamped to avoid settling. In drip lines, create a minimum 1-inch-per-foot positive slope away from foundations for the first 4 feet. Reseed all areas where topsoil was replaced throughout the yard and in play areas. Reseed or reestablish ground cover in foundation plantings. New topsoil must have a lead concentration of less than 400 ppm. Soil cultivation or rototill mixing is not allowed to lower concentration.

NOTE: New topsoil is required to be sampled during clearance.

WALLS AND CEILINGS

REPAIR INTERIOR PLASTER (Interim Control)

Using lead-safe work practices, remove and properly dispose of all damaged or loose plaster. Patch base coat as needed and apply a finish coat using plaster or a plaster patching compound. Remove all excess material and match texture of existing surface.

EDGE/CORNER PROTECTOR (Interim Control)

Using lead-safe work practices, slowly HEPA vacuum areas to be protected. Securely install a corner protector from the floor or top edge of the floor baseboard up not less than 4 feet.

WINDOWS

WINDOW INTERIOR STABILIZE AND PAINT (Interim Control)

Using lead-safe work practices, wet scrape all loose, peeling, cracked, or blistered paint from interior window components, including sashes and sill. Feather edges with a wet sponge sanding block. HEPA vacuum all dust and chips. Wash all surfaces with a de-glossing wash. Apply high-quality bonding primer. Apply high-quality interior latex paint that has a first wet coat film of not less than 6 mils.

WINDOW EXTERIOR STABIZE AND PAINT (Interim Control)

Using lead-safe work practices, repair or replace all areas of damaged or rotten wood. Wet scrape all loose, peeling, cracked, or blistered paint from exterior window components, including sashes, sills, jambs, stops, and trough. Feather edges with a wet sponge sanding block. HEPA vacuum and wash with a de-glossing solution all surfaces to be repainted to ensure a good bond with the new paint. Apply high-quality bonding primer. Apply high-quality exterior latex or acrylic paint that has a first coat, wet film of not less than 6 mils.

WINDOW VINYL REPLACEMENT (Abatement)

Using lead-safe abatement practices, remove and properly dispose of the existing window sashes, parting bead, and outer stops. Supply and install a new vinyl replacement window unit sized to fit the existing opening. Outer stops are to be replaced with new pine or fir using lead-safe work practices or wrapped with aluminum cladding. (Note: Also include specifications to address the interior and exterior trim and casing using either interim controls or abatement.)